Second-order attack on masked AES implementation

Advanced SCA Training

Goal: How to attack a masked implementation of AES

- Applying masking countermeasure to AES

- Measurement setup

- Using second-order attack to defeat masking

- AES is a NIST standardized symmetric key cryptographic algorithm

- Straightforward implementations of AES can be attacked easily using Side-channel attacks (Ex: power, EM)

- Several countermeasures exist to thwart SCA

  - Random delays

  - Shuffling

  - Masking

- Masking is one of the most widely used countermeasure

- Choose an intermediate variable that is function of the secret key (Ex: S-box output)

- Collect the power/EM traces

- Compute the hypothetical intermediate values for each key candidate

- Estimate the hypothetical power consumption (or EM) values using suitable power model

- Compare the hypothetical values with the measured traces using a statistical distinguisher (Ex: DOM. CPA)

- In Masking, we divide each sensitive variable into two shares: mask (generated randomly) and the masked value

- All the operations are performed independently on the two shares

- The first-order DPA/CPA attack involving single point on

- The state variable is divided into two shares: mask and the masked value

- We can perform all the linear operations (KeyAddition, ShiftRows, MixColumns) independently on the shares

- However, it’s not straightforward in case of SubBytes (S- box) due to it’s non-linearity

- In software SubBytes operation is implemented using lookup table for performance reasons

- Solution: Create a masked LUT

- Let the original S-box LUT	be S which is of size 256 bytes

- We need two masks: input mask to mask the index, and output mask to mask the S-box output.

- Let m1 be the input mask and m2 be the output mask

- We create a new look-up table S1	such that S1(x ⊕ m1 ) = S(x) ⊕ m2

- In some cases m1 and m2 can be same, but we use different masks in our implementation

- Masked implementations can still be attacked by combining two points of the trace by correlating the joint distribution with the hypothetical values

- The two points corresponds to the two shares of the sensitive variable (Ex: S-box output)

- There exists several combination functions to combine the points:

  - Absolute difference

  - Product

  - Square of sum

- In a second-order attack the first step is to identify the points on the trace where the two shares are being manipulated

- We do not know exactly where the shares are being manipulated. So we consider a range

- Assume (ti , tj) is the interval corresponding to one share and (tk , tl) corresponds to the other

- We combine every point in (ti , tj) with every point in (tk , tl) using one of the combination function to create a new set of traces

- Choose two shares of the intermediate variable to be attacked (Ex: S-box output; S1(x ⊕ m1 ) = S(x) ⊕ m2 , m2 where x=p ⊕	k).

- Find the interval where m2 is generated and S1(x ⊕ m1 ) is computed and combine using preprocessing step

- For all possible values of x estimate the hypothetical power values by computing the Hamming weight (or Hamming distance, …) of S(x)

- Compute the correlation coefficient between the preprocessed traces and the hypothetical power values

- We can also perform CPA attack directly on 2 bytes thus guessing 216	key candidates at a time

- Consider output of two S-boxes i.e. S1(x1 ⊕ m1 ) = S(x1) ⊕

m2 and S1(x2 ⊕ m1 ) = S(x2) ⊕ m2

- Combine the points corresponding to the leakages from two S-boxes

- The hypothetical values are computed by S1(x1 ⊕ m1 ) ⊕

S1(x2 ⊕ m1 ) for all the possible values of x1 and x2

- The correct key candidate gives the maximum correlation with preprocessed traces

High-end Cortex-m4 CPU which can run at 168MHz Designed as a realistic training target (source code available) We will measure the power consumption at the 3.3V pin

No need for amplifier due to board preparation

- PicoScope 3207B USB Oscilloscope

- Used side-channel: power consumption

- External power supply

- PC2 pin for trigger (before and after decryption)

- FTDI cable for communication

- Riscure Current Probe

- Riscure Inspector Software for acquisition and analysis

- Sampling Frequency: 1 GSa/s

- Number of samples: ~650k for the full operation

- Range: 500mV

- Number of traces acquired: 5000

Masked S-box	10 rounds

Masked S-box	1 round

- Extract the point where output mask m2 is generated

- Extract the points where the Subbytes operation for round 1 is happening (S(xi ⊕ ki ) ⊕ m2 )

- Once we have extracted these points we perform second- order attack

- Combine the points where the mask is generated with 16 points corresponding to 16 S-box outputs

~ ES_Mas eaJ UUK_IGs + Resamp rea:=rs- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - 7 ° l.§_

A

5 *10

4

trace 4999	I A

□	3

2

0

-1

-2

-3	*1 00 D

,.	0	5	10	1 5	20

◄  I	m	I

25	30	35	40	,.

6  *1 0	trace 4999

5

4

3

2

0

-1

-2

-3

-4

0

Out

Trace

Status	Traces	Sma  ples	Data

o *rn

trace 4999

4

j

L

1

0

-1

- L

- j

-4

0

LO

40

oO

Insped or 4.8.3-SABuild vl. O SCA+FI	CJ@

Fil e   Ed it    Pertmhati on   Acq1Uisi  tion   Comp ress  Ali gn    Fil t er   Ana l'ysis  Cry pto   Samp l es  Trac es   User  Tools  Wi n d ow   H el p   P rof il e s

'(	J	T	...	_1j Line s: I 1	1 Traces: I 1	1

AES_Mask ed_lOOK_l Gs  (Sk traces only).trs

*1 0 mV	Channel A 0

20

1 5

10

5

0

-5

-10

-15

..,.

.......--+-t-.......-.--+-t----+-t---+-t------t-t---t-1---t-t---t-t---t-<---t-t---t-<---t-t---t-<t-+----t-t---t-<t-+-.........,, t-+-t-t-t-+-t-t-t-t-t-t-......-t-+-......-----t-+---t-+-+-t--t-+--t-+--t-t--t-+--+-t--t-t--+-t--t-t--+-t--+-t--t-1--+-t---t-t--t-1---t-t---t-t---t-+--t-t---t-+--t-t---t-t---t-+--t-t---t-t-................... , ......... ,

---t-1_ ,..........,....,._.---,µ s

..,.

1 0	20	30	40	50	60	70	80	90	1 00	110	1 20	1 30	14 0	1 50

◄	m	►

*1 00 mV	Channel A 0

2

a

-1

-2	*1 0 µs

0	5	10	1 5	20	25	30	35	40	45	50	55	60

Trace

Status

Reaclly

Traces

available: 50 00 displayed: 0 selected: 0

Samp le s	Value	Data

available:65000 0 (650 µs ) d isplayed:151617 (151.62 µs ) at 2573 (2.573 µs ) se lected: 0 - 0.0,511311 at 112327 (112.83 µs ) 11 15 6B 9B DC 1A 68 5C 85 3C OD FID 32 9D BS 48 OB 52 -41E 26 OF 24 02 EA B5 68 03 05 85 F...

20

*1a mv	Channel AO

20

1 5

10

5

a

. !

J

I

.\

-5

-1 a

-15

- · 1	. I

I

50

45

40

35

30

25

20

1 5

1 0

5

0

-5

-10

-15

-20

-25

-30

-35

trace 1 0-19

-40

0	30	60	90	1 20	1 50

18 0	21 0	240	270	300	330	360	390	420

*1 0

8

7

6

5

4

3

2

1

0

-1

-2

-3

-4

-5

-6

-7

-8

-9

-10

-11

-1 2

-13

-14

-15

-16

-17

-18

-19

-20

-21

-22

-23

-24

-25

-26

-27

-28

-29

-30

-31

-32

-33

-34

-35

-36

-37

-38

-39

-40

-41

-4 2

-43

trace 0-9

3390	..410	..430	..450	..470	..490	..510	..530	..550	..570	..590	..610	..630	..650

# Extract the point corresponding to output mask

trace O

1 5

1 0

5

0

-5

-10

-15

-20

-25

-30

-35

-40

0	2	3	4	5	6	7	8	g	1 0	11	1 2	1 3	14	1 5	16

| I Ae sSe rnndO rderAnalys is o n AES_ as!ked _10 0K_1fi s + Res arn ple d Tri mmed El eginnin gAnd Sbo}j)es + Po intsExtracted + .., • Samples Traces F i rs t : l o F i rs t : l o Nu mbe.r: LS I Nu mb er•: '1196 I Masking type Track bit Key b.. , I C) Simp le _ q_ y r!.9?.9... 1?.J C) Adva nced step 2 0 1 0 2 C) 3, C) 4 C) 5 C) 6 C) 7 C) 8 @ HW XOR dis tance Roun d Key bits Block bits 11- 15 I @ 1 2 9 C) 10 @ 128 C) 192 C) 256 1@ 128 0 192 C) 256 11 Pre vious rnun d key l l I I I • @[R) [§] [!] |  |
| --- | --- |
|  | 11 |

•I

| 5t.ep 1 - Advanced ma s ki ng seaond rd·er A:1&5 an a l ys i s - i n i s hed Ke ybyte O :: OX! A at p Bi t i n :: 1 Ke ybyte 4 :: OxDE Ra t i : 0 . 3500296 |  |  |  |
| --- | --- | --- | --- |
| Status Readly | Traces av ailall le: | 2 displayed: 0 s elected: 0 | Samples Data av ailallle: 17 displayed:17 at O s elected: 0 38 BA BO DF E 76 8E D3 95 J B, A 7 36 83 E6 26 AC 7 92 11 6B 10 96 F9 I) 30 FIE 1 F 27 2 EF C2 E8 |

._I ,.    .&-1. '-    ,l         .&.'l ,l    ..,,._       'I.I I.A.li .&.'1,,;1, ..1. 1.:1.1.&   I.I ' '-       [I I

t

.aL ,1, .&. J 1 f

'I' '!.& .&. !'.:& '- Il l

, [I I ._. ._ ,1, l p.

'!.& I.I ' .f:"'-' 1-'1 ..l. l.l'..&. l..l .li.&. 111

'-" if

3 ,I

I<iey  bytie  candidate ::

4   (0x 04} ,,	val ue ::	0 ., 051 8,,	at   p  s i t i on::

7,I

' I.A   .li   ,l, ' l,,;I,       1.:1. ..1. 1..'1  1.1"1.A.li  .&.!li:.I' "-     Il l    .

1m  d  di st anoe::   1

Ana l yzed ke y bytie 1 5

o,,  Ke y  bytie  can dida.te  ::	6   (0x 06} ,,	val ue ::	0 ., 2326 ,,	a.t   p

siti  n111

11    ,,	and di s t ano e:: 1

Ae sSe rn:nidOrd erAnalysis on AES_Mas!ked _HJOKJ  .:Gs +  Re sam, p le d  · Tri mmed Eegi1rmingA ndSbo )>l)es +  PoimsErtrad ed ,tr:s

Samples

Fri s t:	-1....1        . 	,

Nmu ber: 17_

M asking type

0  mSi  ple	O Ad vanced step 1

Track bit

Traces

First:   l  o

Number:: 1: =1t1=2================================

Key b..,

XOR distance

Ro und

Key bits

2

g	O 10

@ 128	0 192	0 2"56

Block bits

(j;) 128	0 192	0 256

Previous round key

| 1 ,, Ke y | bytie | c.andida.te | :: | 1 | 5 9 | (0x 9F} ,, | val ue :: | 0 ., 0579,, | a.t | p | siti n | ., a,, |  | and di s ta nce :: 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,, I<iey | bybe | C•and ida.te | :: |  | 1 | (0x 0l ) ,1 | val ue :: | 0 ., 055 2,, | a.t | p | siti n | ..,, 11 | ,, | and di s ta noe:: | 1 |
| 3,I I<iey | bytie | candidate | :: |  | 4 | (0x 04} ,1 | val ue :: | 0 ., 0473 ,, | at | p | siti n | ., 11 | ,, | and di s ta noe:: | 1 |
| Anal y:zed | k,ey | bybe 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| o,, I<iey bytie candidate :: 7 (0x 07 } ,, val ue :: 0 ., 1 5 72,, at p siti n., 1 5 ,, and di s t ano e:: 1 1 ,, I<iey byte C•and idate :: 148 (0x 94} ,, val ue :: - 0 ., 0567 ,, at. p s i t .i n..,, a,, and di s ta noe :: 1 2,, I<iey bytie candidate :: 1 (0x 0l } ,, val ue :: - 0 ., 0564,, at p s i t i o n:: 9,, and di st ance :: 1 3 ,, Ke y byte C•and idate :: 4 (0x 04} ,1 val ue :: 0 ., 043 2,, at . p s i t .i n..,, 1 5 ,, and di s t ano e:: 1 M.S ke y:: CA IT BA BE !E BE EF 00 01 02 03 04 05 06 07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Are first-order analyses (e.g. CPA) successful on a masked implementation?

  - No (unless implementation is not proper)

How can we combine two samples for a high-order attack?

  - Mathematical function: absolute difference, product, square of sum…

Which operations need to be masked in a cipher?

  - Non-linear operations (e.g. S-boxes)

Given the trace set from masked AES implementation obtained from Piñata, perform second-order attack to obtain the used secret key.

Try to do it in backwards incremental steps:

  - Try first the attack with the extracted points traceset

  - Afterwards, create a module to extract the points from the trimmed traceset and try repeating the attack

  - Finally, post-process the original traceset to create an aligned trimmed traceset that allows point extraction

Questions or remarks?	,1scura